Institute for Chemical and Bioengineering, Department of Chemistry and Applied Biosciences, ETH Zurich, Vladimir Prelog Weg 1, Zurich 8093, Switzerland.

NCCR Catalysis, Zurich 8093, Switzerland.

- Email: gonzalo.guillen.gosalbez@chem.ethz.ch, jpr@chem.ethz.ch

† Electronic supplementary information (ESI) available: See DOI: 10.1039/x0xx00000x

Received 00th January 20xx,

Accepted 00th January 20xx

DOI: 10.1039/x0xx00000x

Analysis of nitrous oxide as a green oxidant based on economic, environmental, and safety metrics

Abhinandan Nabera,a,b Gian Marco Beshara,a,b Gonzalo Guillén-Gosálbez*a,b and Javier Pérez-Ramírez*a,b

xxx

Introduction

Nitrous oxide, N2O is emerging as a promising and versatile oxidant in heterogeneous catalysis, primarily due to its ability to selectively deliver mono-oxygen species during oxidation reactions. This unique feature minimises overoxidation and by-product formation, making N2O an attractive alternative to conventional oxidants like hydrogen peroxide (H2O2). Moreover, N2O's benign by-product—nitrogen gas—and its favourable properties, such as high solubility in non-polar media and thermal stability, further enhance its industrial appeal. Early breakthroughs, such as the one-step oxidation of benzene to phenol using N2O over Fe-ZSM-5 catalysts, have demonstrated its potential for highly selective transformations. However, the broader application of N2O has been hindered by the risks associated with its conventional production route, i.e., the thermal decomposition of ammonium nitrate (NH4NO3)

Methods

An overview of the methodological framework applied in this study is provided in Fig. 1. This work integrates process modelling, techno-economic analysis, life cycle assessment (LCA), and safety evaluation to systematically compare conventional and emerging routes for N2O, H2O2, and phenol production. These process-based metrics are used to evaluate key performance indicators, including mass and energy balances, production costs, environmental impacts, and safety metrics. By combining simulation-derived data with literature-based parameters, the study provides a comprehensive understanding of the potential benefits associated with each route. The following sections describe the methodology in more detail. For more information on the methodology employed in this work, please refer to the ESI.

Process modelling

The process simulations for N2O, H2O2, and phenol production were developed using Aspen Plus v12.1, employing standard process models to simulate each route (refer to Figs. S1–S5 of the ESI for the process flow diagrams). The mass and energy flows, as well as process unit sizes, which were later used for the economic, environmental, and safety assessments, were extracted from these simulations. For N2O, two routes were analysed: the one-step direct NH3 oxidation and the five-step NH4NO3 decomposition pathway. H2O2 production was modelled via the anthraquinone autoxidation (AO) process, using 2-ethyl anthraquinone as the working solution. Phenol production was analysed through the conventional cumene oxidation route and compared with emerging routes involving direct oxidation of benzene using N2O or H2O2. Mass and energy balances for all processes are summarized in Tables S1–S3 of the ESI. Further details on the process modelling approach are provided in Section 1 of the ESI.

Techno-economic analysis

The techno-economic assessment was based on simulation outputs, which provided essential mass and energy flow data and equipment sizing for each production technology. Operational expenditures (OPEX) and capital expenditures (CAPEX) were estimated to estimate the production cost associated with each production pathway. The 2022 base case used feedstock and utility prices shown in Table S4 of the ESI. For future projections to 2050, a prospective analysis was conducted, acknowledging uncertainties. Table S5 of the ESI details the adjusted feedstock and utility prices used for 2050. A full description of the techno-economic methodology is provided in Section 2 of the ESI.

Life cycle assessment

The life cycle assessment (LCA) followed ISO 14040/14044 standards1,2 and was performed using an attributional cradle-to-gate approach, assuming a functional unit defined as 1 kg of chemical (N2O, H2O2, or phenol). Foreground data were obtained from process simulations, while background data were extracted from the Ecoinvent v3.10 database.3 Climate change impacts (i.e., GHG emissions) were calculated using the IPCC 2021 global warming potentials over a 100-year average.4 Additionally, the ReCiPe 2016 v1.03 method was used to evaluate impacts on human health, ecosystems, and resource depletion.5 Table S6–S8 of the ESI provide life cycle inventories for each chemical production route. For 2050 projections, a prospective LCA was conducted using premise v2.1.3 framework,6 integrated with data derived from Integrated Assessment Models (IAMs). Detailed information on the LCA framework and assumptions can be found in Section 3 of the ESI.

Safety assessment

Safety metrics were evaluated using the Dow Fire and Explosion Index (F&EI) and the trinitrotoluene (TNT) equivalency method.7,8 The Dow F&EI, applied to N2O and H2O2 production technologies, quantifies fire and explosion hazards based on material characteristics, process conditions, and equipment configurations. The Dow F&EI values, presented in Table S9 of the ESI, allow classification into hazard categories (e.g., light, moderate, severe), aiding risk management and insurance planning. Additionally, the TNT equivalency framework was employed to compare the explosion energy potential of various feedstocks used, providing an effective metric for assessing the relative hazard of different production routes (see Table S10 of the ESI). Further details on the safety assessment are presented in Section 4 of the ESI.

Results and discussion

Economic and environmental performance of N2O production pathways from fossil and green NH3

This section evaluates the economic performance and climate change impacts of two N2O production technologies: direct one-step oxidation of NH3 and conventional five-step thermal decomposition of NH4NO3, using both fossil (from natural gas reforming) and green (utilizing electrolytic H2) NH3 as inputs.

As shown in Fig. 2a, the one-step NH3 oxidation route consistently demonstrates lower production costs compared to the five-step thermal decomposition pathway. The total cost of N2O is 0.6 USD kg–1 for the one-step fossil scenario and 1.2 USD kg–1 for the green scenario, compared to 0.9 USD kg–1 and 1.4 USD kg–1 for the corresponding five-step cases. The cost advantage of the one-step route is mainly attributed to its lower electricity consumption, primarily due to minimal pressure fluctuations required in comparison to the five-step scenario. Moreover, the one-step route requires four times lower capital expenditure (CAPEX) in comparison to the five-step route. Notably, NH3 remains the dominant cost drive across technologies, contributing around 85% to the overall costs on average.

Fig. 2b presents the cradle-to-gate climate change impacts associated with each N2O production pathway. Under the fossil-based route, the one-step process has a lower impact of ca. 2.8 kg CO2-eq kg–1 N2O compared to 3.4 kg CO2-eq kg–1 for the five-step route. The lower impacts in the one-step routes are primarily due to reduced electricity use in the process. This trend remains consistent when green NH3 is used, with impacts 0.7 vs. 1.1 kg CO2-eq kg–1 N2O for the one- and five-step processes, respectively. Similar to the production costs, NH3 is the largest contributor to the climate change impacts, especially in the fossil scenarios.

Projections for 2050 under a decarbonized electricity mix (purple diamond in Fig. 2) demonstrate potential reductions in both cost (except for fossil scenarios) and climate change impacts. The climate change impacts of N2O production drop by 10% and 70% on average in the fossil and green scenarios, respectively. The main reductions in the fossil-based scenarios are due to the decarbonized electricity used in the future, while the main reductions in the green scenario are due to both decarbonized electricity and potential efficiency improvements in wind electricity generation, which is used to power the electrolyser unit to produce H2 and subsequently NH3.9 In terms of production costs, future production costs increase for the fossil-based routes by ca. 20%. In contrast, the green route shows reductions of up to 30%, due to increase in efficiency of the electrolyser and lower wind generation costs.

Overall, these findings support the need for prioritizing technological innovation and research efforts in one-step N2O synthesis and accelerating the deployment of low-carbon NH3 production technologies.

Safety performance of N2O production pathways

To complement the techno-economic and environmental assessments, we conducted a quantitative risk assessment of both one- and five-step N2O production technologies using the Dow F&EI and the TNT equivalency method, summarized in Table 1. These frameworks estimate the physical and financial consequences of potential process accidents, providing additional insights into operational safety and business risk.

The one-step process is classified as a light hazard (F&EI = 47). In contrast, the five-step route is rated as a severe hazard (F&EI = 238) due to the explosive nature of the nitrate salts. This results in a much larger exposure area and higher projected property damage and business interruption losses. Furthermore, the TNT equivalency highlights the five-step process’s risk, with a TNT equivalent mass over five times higher than that of the one-step route, leading to longer projectile ranges and projected fatalities in event of an explosion. Thus, beyond its economic and environmental advantages, the one-step route also offers significantly lower safety risks compared to the five-step alternative.

Production costs and climate change impacts of N2O and H2O2 as green oxidants

Next, we compare the one-step N2O production pathway with H2O2 due to their shared characteristics as selective mono-oxygen donors, high oxidation selectivity, and the production of benign by-products, N2 in the case of N2O and water for H2O2. This section compares their economic and environmental performance, considering both fossil and green production pathways.

As shown in Fig. 3a, H2O2 produced via the anthraquinone-based AO process is significantly more expensive than N2O across all scenarios. In the fossil case, H2O2 costs reach 1.6 USD kg–1, approximately 2.6 times higher than N2O (0.6 USD kg–1). Under the green scenario, utilizing electrolytic hydrogen powered by renewable electricity, H2O2 remains 1.7 times more expensive (1.9 vs. 1.2 USD kg–1). The higher costs of H2O2 are largely driven by the 2-ethylanthraquinone working solution. In contrast, for N2O, NH3 feedstock costs are the main contributors, with minimal contributions from utilities and CAPEX.

Furthermore, Fig. 3b presents the climate change impacts for both oxidants. Under fossil-based production, N2O has a higher climate change impact of 2.8 kg CO2-eq kg–1 vs. 1.3 kg CO2-eq kg–1 for H2O2. However, in the green scenario, impacts fall significantly to 0.7 kg CO2-eq kg–1 for N2O and 0.5 kg CO2-eq kg–1 for H2O2. In the green scenario, by 2050, N2O’s impact further drops to 0.3 kg CO2-eq kg–1, slightly outperforming H2O2 (0.4 kg CO2-eq kg–1).

To explore cost-effective pathways for reducing the climate change impact of N2O, we evaluated a blended ammonia scenario combining fossil-based and green NH3 feedstocks. As shown in Fig. 4, a mix of 28% fossil and 72% green NH3 enables N2O production to match the climate change impacts of fossil-derived H2O2 (1.3 kg CO2-eq kg–1). This blended scenario achieves a 38% lower cost compared to H2O2, highlighting a viable strategy that balances environmental performance while being economically competitive. By 2050, further cost reductions are projected for N2O due to improvements in green NH3 production, reinforcing N2O’s competitiveness as a green oxidant.

Quantitative safety evaluation of N2O and H2O2 production

Next, we compare the Dow F&EI and TNT equivalency for the one-step N2O vs. H2O2 production via the AO process, as summarized in Table 2. The one-step N2O process presents a lower overall hazard, with an F&EI of 47 (light hazard), compared to 111 (intermediate hazard) for H2O2. This results in a smaller exposure radius and area for N2O. Similarly, TNT equivalency reveals a stark contrast in explosion potential, with H2O2 having a TNT equivalent mass of 70,748 kg, more than ten times higher than that of N2O, further resulting in a greater fatality risk. Overall, the one-step N2O production pathway poses significantly lower safety risks compared to both the five-step N2O and the H2O2 production routes.

Direct oxidation of benzene to phenol using N2O

To support the case of N2O as a viable oxidant, we evaluate its performance in a key application, i.e., phenol production. Today, phenol is a key intermediate in the chemical industry and is traditionally produced via the conventional cumene oxidation process, which involves the alkylation of benzene with propylene to form cumene, followed by oxidation to yield phenol and acetone. While commercially dominant, the cumene oxidation process has several drawbacks, such as being multi-step, energy-intensive, and requiring complex separations. However, direct oxidation pathways can convert benzene directly to phenol in a single step using selective oxidants like N2O or H2O2.

As shown in Fig. 5a, the direct oxidation of benzene with N2O offers the most cost-effective route to produce phenol. With a cost of 1.1 USD kg–1 in the fossil scenario, it reduces costs by ca. 10% compared to the conventional cumene route. In contrast, H2O2-based oxidation is significantly more expensive, at 1.6 USD kg–1, due to the high cost of the anthraquinone working solution required to produce H2O2. In the green scenario, where N2O and H2O2 are derived from renewable sources, costs increase to 1.3 USD kg–1 and 1.8 USD kg–1, respectively, maintaining N2O’s clear economic superiority.

Fig. 5b compares the cradle-to-gate climate impacts of all routes. The conventional cumene oxidation process shows the highest emissions, at 4.6 kg CO2-eq kg–1 phenol. Direct oxidation pathways significantly reduce the climate change impacts to 4.3 and 3.3 kg CO2-eq kg–1 using N2O and to 3.3 and 2.9 kg CO2-eq kg–1 using H2O2, in the fossil and green scenarios, respectively. By 2050, both alternatives show further reductions in impacts, representing a 10–15% lower carbon footprint on average across scenarios. This case study demonstrates the potential of N2O to serve as a scalable, green oxidant in industrial applications.

Conclusions and outlook

Our work emphasizes that, for N2O production, the direct one-step NH3 oxidation route consistently outperforms the traditional five-step NH4NO3 decomposition process, delivering lower production costs and reduced GHG emissions across both fossil-based and green NH3 scenarios. The one-step route offers roughly one-third lower cost, while also lowering emissions by at least 15% under the fossil scenario. These advantages are mainly attributed to streamlined process design, avoiding energy-intensive intermediate steps. Comprehensive, multi-criteria analysis is crucial to ensure that trade-offs do not occur due to improvements in one dimension. Therefore, in this study we complement techno-economic and environmental assessments with a quantitative safety evaluation, which reveals substantial advantages for the one-step process in terms of operational risk. These combined benefits, i.e., lower cost, reduced emissions, and enhanced safety, highlight the one-step process as a more robust pathway for N2O production.

Benchmarking one-step N2O against H2O2, another key oxidant with similarities such as mono-oxygen donor functionality and benign by-products, highlights critical trade-offs in cost, environmental impact, and safety metrics. Under fossil conditions, H2O2 produced via the AO process exhibits significantly higher costs, ca. 2.6 times that of N2O, primarily due to the expensive anthraquinone working solution. In terms of climate change impacts, H2O2 outperforms N2O in the fossil scenario (1.3 vs. 2.8 kg CO2-eq kg–1), a gap that narrows substantially when green hydrogen from electrolysis is employed. Notably, by 2050, green N2O achieves a lower climate impact than green H2O2 (0.3 vs. 0.4 kg CO2-eq kg–1), reversing the initial trend and underscoring the importance of prospective assessments while evaluating chemical technologies. From a safety perspective, the one-step N2O process demonstrates a lower hazard potential than that of H2O2. These findings position one-step N2O as a competitive oxidant, particularly under future low-carbon scenarios.

The aforementioned advantages of N2O extend beyond its production to key industrial applications. In phenol synthesis, the direct oxidation of benzene using N2O emerges as the most economically and environmentally attractive pathway, outperforming both the conventional cumene oxidation process and H2O2-based direct oxidation of benzene. These findings support the broader potential of N2O as a scalable green oxidant for selective mono-oxygenation reactions.

Overall, the methodological framework applied in this work, linking process simulation with techno-economic analysis, LCA, and quantitative safety metrics, offers a holistic evaluation of emerging chemical technologies. Comprehensive application of this framework to other chemical production systems may uncover pathways that yield benefits across economic, environmental, and safety metrics. The results of this study will aid future advancements in the production and use of green oxidants, in line with green chemistry principles such as atom economy, safer chemical processes, and the use of less hazardous synthesis methods.

Author contributions

AN: methodology, visualisation, formal analysis, writing – original draft, writing – review and editing; GMB: methodology, visualisation, writing – original draft, writing – review and editing; GGG: validation, writing – original draft, writing – review and editing, supervision, and project administration; JPR: conceptualisation, validation, writing – original draft, writing – review and editing, supervision, and project administration.

Conflicts of interest

There are no conflicts to declare.

Data availability

The data presented in the figures of this paper are publicly available via Zenodo at xxx (ref. xx). The background LCI datasets used in this study are available in the Ecoinvent v3.10 (cut-off system model) database under the accessible link (https://ecoinvent.org). Other supporting data are available from the corresponding authors upon request.

Acknowledgements

This publication was created as part of NCCR Catalysis (grant number 225147), a National Centre of Competence in Research funded by the Swiss National Science Foundation.

References

- 1	International Standards Organization, In ISO 14040:2006 Environmental Management--Life Cycle Assessment--Principles and Framework, 2006.

- 2	International Standards Organization, In ISO 14044:2006 Environmental Management--Life Cycle Assessment--Requirements and Guidelines, 2006.

- 3	G. Wernet, C. Bauer, B. Steubing, J. Reinhard, E. Moreno-Ruiz and B. Weidema, Int. J. Life Cycle Assess., 2016, 21, 1218–1230.

- 4	Intergovernmental Panel on Climate Change (IPCC), Ed., in Climate Change 2022 - Mitigation of Climate Change, Cambridge University Press, 1st edn., 2023, pp. 1727–1790.

- 5	M. A. J. Huijbregts, Z. J. N. Steinmann, P. M. F. Elshout, G. Stam, F. Verones, M. Vieira, M. Zijp, A. Hollander and R. van Zelm, Int. J. Life Cycle Assess., 2017, 22, 138–147.

- 6	R. Sacchi, T. Terlouw, K. Siala, A. Dirnaichner, C. Bauer, B. Cox, C. Mutel, V. Daioglou and G. Luderer, Renew. Sustain. Energy Rev., 2022, 160, 112311.

- 7	American Institute of Chemical Engineers, Dow’s Fire & Explosion Index Hazard Classification Guide, John Wiley & Sons, Inc, Hoboken, 7th ed., 2010.

- 8	D. A. Crowl and J. F. Louvar, Chemical process safety: fundamentals with applications, Prentice Hall, Upper Saddle River, NJ, 3rd ed., 2011.

- 9	A. Nabera, A. José Martín, R. Istrate, J. Pérez-Ramírez and G. Guillén-Gosálbez, Green Chem., 2024, 26, 6461–6469.