Fig. 1 Outline of the analysis. First, we compare the five-step and one-step N2O production routes using economic, environmental, and safety metrics (Fig. 2 and Table 1). In the same analysis, we show how emissions can be further reduced using green NH3, i.e., NH3 produced via water electrolysis powered by clean electricity. Next, we extend our analysis to compare N2O with H2O2, another mono-oxygen donor with benign by-products, again in terms of cost, environmental impact, and safety metrics (Figs. 3 and 4, and Table 2). Finally, we explore key applications for N2O, such as the direct production of phenol from benzene oxidation, and compare this route with both the direct oxidation of benzene using H2O2 and the traditional cumene-based process for phenol production (Fig. 5). aThe reaction of NO2 with water to produce HNO3 is depicted here as a simplified representation of nitrogen oxides absorption; in reality, NO2 forms N2O4, which hydrolyses to HNO3 and NO.

Fig. 2 (a) Production costs and (b) climate change impacts per kg of N2O produced using two technologies: one-step NH3 oxidation and five-step thermal decomposition of NH4NO3. The analysis includes N2O production via both fossil-based NH3 (derived from natural gas) and green NH3 (produced via the electrolytic H2-based Haber-Bosch process). Future projections for 2050 are highlighted for both the production costs and climate change impacts across all technologies.

- The one-step direct oxidation of NH3 to N2O performs better than the conventional five-step process of NH3 to N2O via NH4NO3 decomposition in terms of both production costs and climate change impacts.

- NH3 is the main contributing factor to both production costs and climate change impacts. However, CAPEX contributes more to production costs, and electricity contributes more to climate change impacts in the five-step process compared to the one-step direct oxidation technology.

Table 1 Risk assessment metrics for the one-step and five-step N2O production routes are evaluated using the Dow Fire and Explosion Index and the TNT equivalency framework, highlighting the estimate financial and operational impacts.

| Metric | 1-step | 5-step |
| --- | --- | --- |
| Dow Fire and Explosion Index |  |  |
| Fire and explosion index | 47 | 238 |
| Degree of hazard | Light | Severe |
| Radius of exposure (m) | 12 | 61 |
| Area of exposure (m2) | 464 | 11678 |
| Maximum probable property damage (million USD) | 2 | 32 |
| Maximum probable days outage | 28 | 166 |
| Business interruption (million USD) | 11 | 89 |
| Trinitrotoluene (TNT) equivalency |  |  |
| Equivalent mass of TNT (kg) | 6008 | 30727 |
| Overpressure (kPa) | 49 | 142 |
| Number of deaths due to lung haemorrhage | 0 | 5 |
| Number of eardrum ruptures | 1 | 3 |
| Maximum distance at which projectiles can land (m) | 1920 | 2737 |

Fig. 3 (a) Production costs and (b) climate change impacts per kg of N2O and H2O2. For N2O production, the one-step process using NH3 derived from either natural gas (fossil-based) or the electrolytic H2-based Haber-Bosch process (green) is shown. Similarly, the anthraquinone process for H2O2 production, utilizing H2 from either natural gas reforming or electrolytic sources, is presented. Future projections by 2050 for production costs and climate change impacts across all technologies are indicated.

- Hydrogen peroxide (H2O2) is significantly more expensive than N2O alternatives (2.6 times in the fossil scenario and 1.7 times in the green scenario) due to the costly anthraquinone working solution required for the process.

- Regarding climate change impacts, in the fossil scenario, N2O has an impact of 
2.8 kg CO2-eq kg–1, compared to H2O2’s impact of 1.3 kg CO2-eq kg–1. In the green scenario, these impacts decrease to 0.7 kg CO2-eq kg–1 for N2O and 0.5 kg CO2-eq kg–1 for H2O2. The fossil N2O production process exhibits significantly higher impacts than H2O2 production, primarily due to the hydrogen (H2) required (2.3 times more in the case of N2O than in H2O2).

- In the future, as electricity becomes highly decarbonized, the impacts of N2O in the green scenario decrease to 0.3 kg CO2-eq kg–1, while H2O2 remains at 0.4 kg CO2-eq kg–1.

Fig. 4 Production costs of N2O and H2O2 are compared under the condition of having the same carbon footprint. To match the climate change impact of fossil-based H2O2, a blend of N2O—comprising 28% fossil-derived and 72% green NH3 (see Fig. 3)—is utilized. By 2050, projections indicate potential reductions in production costs for N2O.

- A blending scenario between fossil and green NH3 for N2O production suggests that, at a blend ratio of 28% fossil and 72% green NH3, the carbon footprint of N2O production can match that of fossil-based H2O2 production at a 38% lower production cost.

Table 2 Risk assessment metrics for one-step N2O production compared to H2O2 production via the conventional anthraquinone process are evaluated using the Dow Fire and Explosion Index and the TNT equivalency framework, highlighting the estimate financial and operational impacts.

| Metric | N2O | H2O2 |
| --- | --- | --- |
| Dow Fire and Explosion Index |  |  |
| Fire and explosion index | 47 | 111 |
| Degree of hazard | Light | Intermediate |
| Radius of exposure (m) | 12 | 28 |
| Area of exposure (m2) | 464 | 2546 |
| Maximum probable property damage (million USD) | 2 | 2 |
| Maximum probable days outage | 28 | 30 |
| Business interruption (million USD) | 11 | 12 |
| Trinitrotoluene (TNT) equivalency |  |  |
| Equivalent mass of TNT (kg) | 6008 | 70748 |
| Overpressure (kPa) | 49 | 264 |
| Number of deaths due to lung haemorrhage | 0 | 9 |
| Number of eardrum ruptures | 1 | 4 |
| Maximum distance at which projectiles can land (m) | 1920 | 3280 |

Fig. 5 (a) Production costs and (b) climate change impacts per kg of phenol produced using the business-as-usual cumene oxidation process, and the direct oxidation of benzene to phenol using N2O and H2O2. For N2O contributions, the one-step process using NH3 derived from either natural gas (fossil-based) or the electrolytic H2-based Haber-Bosch process (green) is shown. Similarly, the anthraquinone process for H2O2 contributions, utilizing H2 from either natural gas reforming or electrolytic sources, is presented. Projections for 2050 highlight potential reductions in production costs and climate change impacts across all technologies.

- The direct oxidation of benzene to phenol using N2O is the most cost-effective route, reducing costs by 10% compared to the traditional cumene oxidation process for phenol production.

- In terms of climate change impacts, all direct oxidation routes have lower impacts than the traditional cumene oxidation process. Among these, the H2O2-based direct oxidation routes exhibit the lowest impacts for phenol production.