Green Foundation Box

1 | How does your work advance the field of green chemistry?

By integrating the three pillars of sustainability—economic viability, environmental performance, and safety—our study establishes a holistic, metrics-based framework for evaluating emerging chemical technologies. Applied to nitrous oxide (N2O), the analysis highlights its potential as a selective oxidant with lower cost, reduced environmental footprint, and improved safety relative to conventional alternatives, while offering a transferable template for the assessment of other sustainable processes

2 | Can you describe your specific green chemistry achievement, either quantitatively or qualitatively?

We show that the one-step ammonia oxidation to N2O cuts costs and carbon footprint by over 20% while improving safety versus the conventional five-step ammonium nitrate route. Benchmarking against hydrogen peroxide, H2O2, a well-established oxidant, N2O from a fossil/green ammonia blend offers equivalent carbon footprint at ~40% lower cost and hazard potential. We also highlight N2O’s potential as a green oxidant in phenol production.

3 | How could your work be made greener and be elevated by further research?

The benefits of N2O production rely on promising catalysts demonstrated only at the laboratory scale. Scaling them to industrial levels could realise the potential of the one-step route by lowering costs, emissions, and safety risks, unlocking its promising future for selective oxidations.