Fig. 1 | a, Production costs and b, climate change impacts per kg of N2O and H2O2. For N2O production, the one-step process using ammonia derived from either natural gas (fossil-based) or the electrolytic hydrogen-based Haber-Bosch process (green) is shown. Similarly, the anthraquinone process for H2O2 production, utilizing hydrogen from either natural gas reforming or electrolytic sources, is presented. Future projections for 2050 of production costs and climate change impacts for all technologies are indicated to illustrate the potential reductions.

- In fossil-based production, H2O2 is significantly more expensive than N2O alternatives (2.4 times in the fossil scenario and 1.6 times in the green scenario) due to the costly anthraquinone working solution required for the process.

- By 2050, the green N2O process demonstrates a 41% reduction in production costs, while the H2O2 process shows less than a 10% reduction.

- Regarding climate change impacts, in the fossil scenario, N2O has an impact of 4.4 kgCO2e per kg, compared to H2O2’s impact of 1.3 kgCO2e per kg. In the green scenario, these impacts decrease to 1.9 kgCO2e per kg for N2O and 0.5 kgCO2e per kg for H2O2. The green N2O production exhibits much higher impacts than H2O2 due to the cryogenic cooling required to separate unreacted oxygen.

- However, in the future, as electricity becomes highly decarbonized, the impacts of N2O in the green scenario decrease to 0.1 kgCO2e per kg, while H2O2 remains at 0.3 kgCO2e per kg.

- The contribution of catalysts (Au for N2O and Pd for H2O2) has a negligible effect on both production costs and climate change impacts.